# Computer Diagnostic Checklist

Professional working template

| Work Order # | ____________ |
| --- | --- |
| Device | ____________________________ |
| Reported Issue | ____________________________________________ |

# Pre-Diagnostic Protection

☐ Confirm customer authorization and repair scope.

☐ Record device condition and accessories.

☐ Determine whether data backup is required before invasive work.

☐ Do not alter or erase data until the repair path is understood.

# Power / Hardware

☐ Verify AC adapter/PSU and power indicators.

☐ Inspect for swelling, liquid damage, damaged ports, loose cables, dust, or overheating.

☐ Check CPU/GPU temperatures and fan operation.

☐ Test memory when symptoms indicate RAM instability.

☐ Review storage SMART/health information.

☐ Check battery health where applicable.

# Operating System

☐ Verify boot behavior and startup time.

☐ Check free disk space and resource utilization.

☐ Review Device Manager for driver/device problems.

☐ Run Windows Update status review.

☐ Use SFC/DISM only when appropriate and record commands/results.

☐ Review relevant Event Viewer errors/warnings.

# Network

☐ Record IP configuration, gateway, and DNS behavior.

☐ Test local gateway and internet reachability.

☐ Test name resolution.

☐ Compare wired vs. wireless behavior when relevant.

☐ Capture packets only when necessary and avoid exposing private data.

# Security

☐ Verify Defender/approved AV status.

☐ Verify Windows Firewall status.

☐ Review suspicious startup/persistence items when indicated.

☐ Check browser proxy/DNS configuration when malware is suspected.

☐ Verify Secure Boot/TPM/encryption status when part of service.

# Diagnosis & Recommendation

☐ Root cause or most likely cause documented.

☐ Evidence supporting diagnosis recorded.

☐ Repair recommendation and alternatives documented.

☐ Estimate/approval obtained before exceeding authorization.

# Findings

| Evidence | ________________________________________________________________________________ ________________________________________________________________________________ |
| --- | --- |
| Diagnosis | ________________________________________________________________________________ ________________________________________________________________________________ |
| Recommended Action | ________________________________________________________________________________ ________________________________________________________________________________ |